Artiq Parameters control:

- EC1

- EC2

- Comp_hor

- Comp_ver

- U_rf

- U_dc

- DDS

- Peizo effect voltage

- Be oven shutter

- Bephi shutter

- UV3 shutter

- Piezo effect valve shutter

- Egun shutter

- B-field shutter

Modules:

- Be+ detection

  - Real time Be+ position, sig, area, intensity readout via flask

  - Real time PMT analysis

- Scan module

  - Read current parameter

  - Predict voltage

  - Secular scan

  - Compare

- Micromotion detection (temperature readout)

- Egun optimising algorism

To implement the "Sidecar" (Parallel Loop) architecture effectively, here are the specific documents you need to study. I have categorized them by which part of the system they control.

1. The Architecture (The "Glue")

This is the most critical part. If you do not understand this pattern, your LabVIEW program may freeze while waiting for the Flask server to reply.

- Concept: Producer/Consumer Design Pattern

  - Why you need it: This allows your "Experiment Loop" (Producer) to run fast and uninterrupted, while the "Network Loop" (Consumer) handles the slow communication with Flask.

  - Official NI Documentation: Producer/Consumer Architecture in LabVIEW

  - Key Function to Study: Queues (specifically Enqueue Element and Dequeue Element). This is how you pass data safely between the two loops.

2. The LabVIEW Client (The "Talker")

These are the tools you use to send and receive messages. You will find these inside the Data Communication > Protocols > HTTP Client palette.

- Concept: HTTP Client VIs

  - Why you need it: To actually perform the GET (ask for command) and POST (send data) actions.

  - Official NI Documentation: HTTP Client VIs

  - Specific VIs to look up:

    - OpenHandle.vi (Starts the session)

    - GET.vi (Reads commands from Flask)

    - POST.vi (Sends experiment data to Flask)

    - AddHeader.vi (Required to tell Flask you are sending JSON: Content-Type: application/json)

3. The Translator (The "Language")

LabVIEW variables (Clusters) and Python variables (Dictionaries) are different. JSON is the common language they both speak.

- Concept: Flatten/Unflatten JSON

  - Why you need it: To convert your LabVIEW Cluster into text that Python can understand, and vice versa.

  - Official NI Documentation: Flatten To JSON Function

  - Pro Tip: If you find the native LabVIEW JSON functions restrictive (they can be picky about strict typing), look for the JKI JSON Serialization library in the VI Package Manager. It is a community standard often used for this exact task.

4. The Flask Server (The "Listener")

You only need the basics here. Do not get bogged down in "Template Rendering" or "HTML"—those are for making websites for humans. You are making an API for a machine.

- Concept: Routing and Request Objects

  - Why you need it: To create the specific URLs (/command and /data) and read the JSON data LabVIEW sends.

  - Official Flask Documentation: Flask Quickstart - A Minimal Application

  - Specific Sections to read:

    - Routing: How to map a function to a URL.

    - Accessing Request Data: Specifically how to use request.json to read what LabVIEW sent.

Next Step: Would you like to see the pseudocode for the "Comms Loop" (the radio operator) to see exactly how it uses the Queue to talk to your existing experiment code?