Here is the summary report of the finalized system architecture for your Ion Trap experiment.

1. High-Level Architecture: The "Master Node" Model

The system has shifted from a decentralized analysis model to a centralized "Smart Master" model. The Camera PC now acts as the central brain, handling control, acquisition, and analysis to minimize latency.

- Camera PC (Master Node): The orchestrator. It runs the control algorithm (TuRBO), interfaces directly with the camera hardware, performs real-time image analysis, and issues commands to other devices.

- ARTIQ & LabVIEW PCs (Workers): These function as "remote actuators." They listen for commands, execute hardware tasks (e.g., setting voltages, running pulse sequences), and return sensor data.

- Storage Server (Archive): A passive repository for long-term storage of images (.jpg) and logs (.json, .jsonl). It is removed from the critical real-time feedback loop.

2. Communication Protocols

The system uses a split-plane communication strategy to ensure that heavy data transfer does not block fast control signals.

| Plane | Protocol | Usage |
| --- | --- | --- |
| Control Signals | ZeroMQ (ZMQ) | Used for low-latency command & control (sub-millisecond). The Master Publishes (PUB) parameters; Workers Subscribe (SUB). |
| Time-Series Data | InfluxDB | A real-time database running on the Master Node. Accepts high-frequency sensor readings (Pressure, PMT counts, Ion positions) from all devices simultaneously. |
| Heavy Data | SMB / NFS | Network Attached Storage (NAS) mapped as a drive (Y:/ or Z:/). Used only for archival of images and daily logs. |
| Inter-Process | Localhost ZMQ | Used internally on the Camera PC to let the Control Algorithm talk to the Image Streamer process. |

3. Data Workflow: The Optimization Loop

The data flow is designed for Zero-Latency Feedback, performing all critical steps in RAM on the Master Node before archiving.

- Decision (Master): The TuRBO algorithm calculates the next set of experimental parameters (e.g., Frequency 102 MHz, Voltage 5.0 V).

- Command (Network): The Master publishes these parameters via ZMQ.

- Execution (Workers):

  - LabVIEW receives the message and sets DC voltages.

  - ARTIQ receives the message, configures the DDS, and sends a TTL Trigger to the camera.

- Acquisition (Master): The Camera driver captures a frame directly to RAM.

- Fast Analysis (Master):

  - The raw frame is converted to a numpy array.

  - Your Numba-optimized ROI fitter calculates ion positions and widths ($\sigma$) in <5ms.

- Feedback (Master): The analysis result is fed immediately back into the TuRBO algorithm to generate the next step.

- Archival (Background): A separate child process (camera_streamer.py) saves the visualization (.jpg) and data (.json) to the Storage Server asynchronously, ensuring the main loop never waits for disk I/O.

4. Software Structure

- master_node.py: The main controller. It manages the experiment state, runs the optimization logic, and launches the streamer.

- camera_streamer.py: A robust Child Process managed by the Master. It handles the "boring" work of file watching and saving, isolating the Master from potential crashes or lag.

- image_handler_opt.py: The specialized high-performance analysis engine. It detects file types (.jpg vs .dat) and applies the Hybrid Gaussian/SHM model.

- lab_comms.py: A shared library used by all PCs (Master, ARTIQ, LabVIEW) to ensure they all speak the same JSON data schema.

This architecture provides the speed required for complex optimization algorithms like TuRBO while maintaining the reliability of a distributed logging system.