Since you are dealing with a "heat map" created by long-exposure Simple Harmonic Motion (SHM), your data has a very specific mathematical structure.

The best solution to extract Amplitude ($A$) and Intensity ($I_{total}$) is a three-step algorithmic pipeline: Collapse $\rightarrow$ Fit $\rightarrow$ Extract.

Here is the step-by-step algorithm:

Step 1: Dimensional Reduction (The "Collapse")

You have a 2D image, but the motion happens along only one axis (let's assume the horizontal X-axis).

- Action: Sum the pixel values of every column vertically.

- Result: This converts your 2D image into a 1D Intensity Profile (a single curve).

- Why: This drastically reduces processing time and significantly improves the Signal-to-Noise Ratio (SNR) by averaging out sensor noise.

Step 2: The "Bathtub" Fit (Curve Fitting)

Do not use a Gaussian fit; it will fail because your signal is bimodal (brightest at the edges). Instead, use the SHM Probability Density Function.

You should fit your 1D profile to this theoretical model:

$$I(x) = B + \frac{C}{\sqrt{A^2 - (x - x_0)^2}}$$

Where:

- $x$: The pixel position.

- $x_0$: The center of the motion (equilibrium point).

- $A$: The Amplitude (what you are looking for).

- $C$: A scaling factor related to the Intensity.

- $B$: Background noise level.

Note: In practice, real-world data will look like this theoretical curve smoothed out (convolved) by the width of the sphere.

Step 3: Parameter Extraction

Once the algorithm fits the curve to your data, you get your answers directly from the variables:

1. To find the Amplitude:

Read the parameter $A$ from your fit.

- Alternative (Simpler/Noisier): Just find the pixel indices of the two brightest peaks and calculate: $\frac{\text{Peak}_2 - \text{Peak}_1}{2}$.

2. To find the Intensity:

Calculate the Integral (Area under the curve) of your 1D profile.

- Summing all pixel values in the 1D profile gives you the total photons collected.

- If your exposure time is constant across all images, this Sum is directly proportional to the reflectivity/intensity of the kugel.

Summary of the Algorithm Logic

| Step | Technique | Advantage |
| --- | --- | --- |
| 1. Pre-processing | Vertical Summation (axis=0) | Removes 2D noise; simplifies the problem to 1D. |
| 2. Analysis | Non-linear Least Squares Fit (using the Inverse Square Root function) | Uses all data points to find the edges, making it sub-pixel accurate. |
| 3. measurement | Integration of the curve | Gives the most stable measurement of total light intensity. |

Comparison of Approaches

- Naive Approach (Edge Detection): You look for the first and last non-zero pixel.

  - Risk: Extremely sensitive to background noise. Will likely overestimate amplitude.

- Standard Approach (Peak Finding): You look for the two max brightness points.

  - Risk: Limited to integer pixel precision.

- Best Approach (Model Fitting): You fit the "Bathtub" equation mentioned above.

  - Benefit: It averages out noise over the whole image and can give you the amplitude to a fraction of a pixel (sub-pixel resolution).

Next Step

Would you like me to generate a Python script (using numpy and scipy.optimize) that takes an image, performs this vertical collapse, and fits the specific SHM equation to output your Amplitude and Intensity?